SST Assignment

VIP – Team Phoenix

Fall 2025

Due Date: October 3, 2025

** If you were unable to compile SST from source, there is a build script in Discord that will compile SST and all its dependencies.

Assignment:

- Review slides shown in class on 09/24/25, specifically those that go over the various sections of an SST simulation input file.

  - Feel free to have the demo_1.py script open as you go through the slide deck, because the snippets of the script are from demo_1.py.

- Run the demo_1.py script for SST.

  - This is a simple 1 CPU, cache, and memory simulation provided by the SST devs as part of their tutorial program.

  - Runtime should take <7 µs

- If you did not add the stats commands to your script, be sure to include them and re-run demo_1.py.

- Familiarize yourself with the CSV outfile to answer the questions below. It may help to open it in Excel until you feel comfortable parsing CSVs on the command line.

-------------------------------------- QUESTIONS ON NEXT PAGE --------------------------------------

Questions:

- Report total read and write requests.

  - Using the ‘req_latency’row, calculate the average latency. Hint: (sum of latencies / total requests) = X

- What were the total number of cache hits and misses?

  - Report cache hit rate.

- What is the maximum number of requests per cycle the CPU is allowed to issue? Hint: look in the py script.

  - Calculate the actual requests per CPU cycle.

- How many requests were received by the memory?

  - Calculate average latency.

  - Calculate average outstanding requests.

    - What does the average outstanding request tell us?

“Big Picture” Short Answer

- How did the CPU perform in terms of its requests per cycle vs its maximum number of requests it could have theoretically sent?

- From your calculated cache hit rate, what % of requests leaked to memory? How effective was the cache in reducing memory traffic?

- What do the memory latencies and outstanding requests reveal about the main memory’s role?

- Are you able to identify a system bottleneck?